| Situation Based on pre-1978 unit and assuming positive XRF reading on surface. | Lead Hazard per HUD and NJ Code | Lead Hazard per 8:51 (Abatement Code) |
| --- | --- | --- |
| An INTACT friction surface with dust level above HUD/EPA | YES Strip to bare wood or remove and replace. | YES |
| An INTACT friction surface with dust level below HUD/EPA. (HUD Guidelines 5-28 and 29) | NO However best practice is to strip to bare wood or remove and replace if possible. | YES |
| A deteriorated friction surface with dust level above HUD/EPA | YES Strip to bare wood or remove and replace. | YES |
| A deteriorated friction surface with dust level below HUD/EPA | YES All deteriorated LBP is considered a hazard. | YES |
| An INTACT impact surface with or without dust above HUD/EPA | NO | No, but in practice it may be required. |
| A deteriorated impact surface with or without dust above HUD/EPA | YES | YES |
| “Chewable” surface with no signs of teeth marks or no child under 6 in the home. (HUD Guidelines 5-32) | NO But best practice is to remove/replace or strip to bare wood and encapsulate. | NO, but in practice it is being required. |